“COMPACT” CRANE RADIO SYSTEM

To meet the need for a more compact safe area crane radio system a unit has been designed and manufactured that can be installed in the smallest cab.

The new compact unit uses Motorola radios with a standard single remote control head.

Using licensed Motorola software an interface has been developed to allow ALL functions to be initiated from this control head. Switching from VHF to UHF, the display on the control head indicating to which radio it is connected. Both radios continue to be monitored irrespective of which radio is selected, by means of each radio having its own dedicated speaker. In addition, the optional PA is enabled from this control head again indicating that the PA has been selected on the display. No external switch boxes are needed.

The main equipment is mounted in an IP66 enclosure.

The system is supplied with, footswitch or push button, for radio PTT / PA operation, gooseneck microphone. Headset option available.

Although supplied with Motorola GM360 radios the system can be adapted to accommodate other Motorola professional models.

Other configurations are available to suit customer’s requirements.

##### Technical Specifications

###### RADIOS			VHF					UHF

Frequency Range		136MHz-174MHz			403MHz-470MHz

Channel Spacing		12.5/20/25kHz			12.5/20/25kHz

Transmit Power		1-25 watts				1-25 watts

Receive Sensitivity		<0.3uV (12dB SINAD)		<0.3uV (12 dB SINAD)

###### MAIN ENCLOSURE

Contains		Radios, Power supplies, PA unit and Control interfaces.

Dimensions		300mm x 300mm x 200mm

IP Rating		IP66

Power supply		86-260v AC universal input.

DC options		12 volts, 24 volts, 48 volts

CONTROL UNIT

Dimensions		200mm x 120mm x 80mm

Din Connector for Headset

U Bracket mounting

IP Rating		IP54

PA UNIT (optional)

###### Output Power		25 watts nominal

Output impedance		100 volt line